SPECIFICATIONS                                                                                                                                Domestic Pump Centriflo Series C35 and C17

SPECIFICATIONS                                                                                                                                Domestic Pump Centriflo Series C35 and C17

. . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . .

. . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . .

Domestic® Pump Centrifo® Series C35TM and C17 TM

PART 1 GENERAL

1.01       SECTION INCLUDES

# Unit shall be a Domestic Centriflo  series C35 or C17, style PF or PVF centrifugal pump as manufactured by Bell & Gossett, a Xylem brand.

# Furnish and install an extended life pump with a capacity as indicated in the plans.

1.02       RELATED SECTIONS

- Section 23 22 23 – Steam Condensate Pumps

- Section 23 22 23.13 -  Electric Driven Condensate Pumps

- Section 23 22 23.23  – Pressure Powered Steam Condensate Pumps

1.03       REFERENCES

- HI - Hydraulic Institute.

- ANSI - American National Standards Institute.

- NEMA - National Electrical Manufacturers Association.

- UL - Underwriters Laboratories.

- ETL - Electrical Testing Laboratories.

- CSA - Canadian Standards Association.

- NEC - National Electric Codes.

- ISO - International Standards Organization.

- IEC - International Electrotechnical Commission.

1.04       SUBMITTALS

- Submit data cover sheet.

- Unit description sheet.

- Dimensional print(s).

- Sales bulletin.

- Piping diagram(s).

- Wiring diagram(s).

- Installation, operation & maintenance manual.

1.05      QUALITY ASSURANCE

- The manufacturer shall have a minimum of 30 years experience in the design and construction of condensate return equipment.

- The pump manufacturer shall be fully certified by the International Standards Organization per ISO 9001. Proof of this certification shall be furnished at the time of submittal.

# The manufacturer shall carry a minimum product liability insurance of $5,000,000.00 per occurrence.

# The pump manufacturer shall test according to the ANSI/Hydraulics Institute Standard 14.6, Grade 1, for Rate of Flow, Total Head and Power Consumption.

PART 2 PRODUCTS

2.01      MANUFACTURERS

# Subject to compliance with these specifications, the following manufacturers shall be acceptable:

# Domestic Centriflo Series C35 or C17, manufactured by Bell & Gossett, a Xylem brand.

- Pre-approved equal.

2.02       COMPONENTS.

# Water pump

Items in parentheses denote optional items.

# The water pump shall be a Centriflo Series C17 or C35 bronze fitted, centrifugal design pump, close-coupled to a 1750 RPM or 3500 RPM motor, permanently aligned, and flange mounted for vertical operation.

# The pump shall include:

# One cast iron volute with:

# One discharge gauge port tapping. (Not available for pump size 609PF).

# One drain tapping.

# One dynamically balanced enclosed bronze centrifugal impeller.

# One renewable bronze wearing ring.

# One stainless steel shaft.

# Carbon/ceramic/stainless steel mechanical seal suitable for 250oF (121C) operation.

# The motor shall meet NEMA design specifications with wetted portion stainless steel shaft and shall be the size, voltage, insulation class, duty rating and enclosure called for in the plans.

# Capacities and electrical characteristics for the pump shall be scheduled on the drawings.

PART 3 EXECUTION

  - INSTALLATION

- Install equipment in accordance with manufacturer’s instructions.

- Power wiring, as required, shall be the responsibility of the electrical contractor. All wiring shall be performed per manufacturer’s instructions and applicable state, federal and local codes.

- All factory wiring shall be numbered or coded for easy identification and the numbers shall coincide with those shown on the wiring diagram.

- The unit shall be factory tested as a complete unit and the unit manufacturer shall furnish elementary and connection-wiring diagrams and piping diagrams. Installation and operation instructions shall also be provided.

- The unit shall be shipped completely assembled.

- The factory shall provide a certified test report.

- Unit shall be a Domestic Centriflo Series C35 or C17, as manufactured by Bell & Gossett, a Xylem brand, Morton Grove, IL.

###### END OF SECTION

www.bellgossett.com

Bell & Gossett is a trademark of Xylem Inc. or one of its subsidiaries.

© 2016 Xylem Inc.  Domestic Pump Centirflo Series C35 and C17 Specifications

www.bellgossett.com

Bell & Gossett is a trademark of Xylem Inc. or one of its subsidiaries.

© 2016 Xylem Inc.  Domestic Pump Centirflo Series C35 and C17 Specifications